# FECS Software Design Specification

## 1. Project Overview

Project Name: Forest Emergency Communication System (FECS)

Purpose: FECS is an offline emergency communication system for forest and remote terrain environments. It allows a user module carried by trekkers or forest personnel to send alert messages through LoRa repeater nodes to a base station. The base station is connected to a laptop/server that runs the backend and dashboard application.

The application layer is responsible for receiving alerts, validating them, removing duplicates, storing valid records, showing them on a live dashboard, and triggering acknowledgments back to the user module through the base station.

## 2. High-Level Architecture

### 2.1 Physical Modules

#### 1. User Module

- Contains push button, GPS, LoRa, ESP32, LED, buzzer

- Sends alert messages based on button interaction

- Receives acknowledgment from the base station through relay nodes

#### 2. Relay Module

- Works as a repeater/forwarding node

- Receives LoRa packets and retransmits them

- Extends network coverage between user module and base station

- Does not perform backend logic

#### 3. Base Station Module

- Connected to laptop/server

- Receives messages from user modules through relay nodes

- Forwards structured data to backend

- Receives ACK commands from backend and transmits ACK back through relay nodes to user module

### 2.2 Logical Data Flow

User Module -> Relay Module(s) -> Base Station Module -> Backend -> Database -> Dashboard
Backend -> Base Station Module -> Relay Module(s) -> User Module

## 3. Application Scope

The software application includes: - Backend service using FastAPI - Frontend dashboard using React.js - Live map using Leaflet - PostgreSQL database for alert storage - MQTT-based communication between base station and backend

## 4. Technology Stack

### Backend

- FastAPI

- Pydantic validation

- MQTT client

- PostgreSQL driver / ORM

### Frontend

- React.js

- Leaflet for map visualization

- Real-time updates from backend

### Database

- PostgreSQL

### Communication

- MQTT between base station module and backend application

## 5. Functional Requirements

### 5.1 Alert Reception

The system shall receive messages from the base station module when a user module sends an alert.

### 5.2 Message Types

Supported message types: - normal - high - emergency - cancel

### 5.3 Signal Types

Supported signal types: - manual - auto

### 5.4 Duplicate Removal

The system shall discard duplicate messages and store only the first valid message.

### 5.5 Acknowledgment Trigger

The backend shall trigger ACK only for valid manual alerts: - normal - high - emergency

ACK shall not be triggered for: - auto signals - cancel messages

### 5.6 Dashboard Display

The dashboard shall display valid alerts on a live interface with location markers on a map.

### 5.7 Status Management

Each stored alert shall maintain a status value: - pending - acknowledged - resolved

## 6. Message Communication Design

### 6.1 Incoming JSON Message (Base Station -> Backend)

{
  "packet_id": "a1b2c3d4",
  "device_id": "DEV_001",
  "latitude": 10.934512,
  "longitude": 78.121245,
  "message_type": "emergency",
  "signal_type": "manual",
  "event_time": "2026-03-18T10:32:15Z",
  "source": "hardware"
}

### Field Definitions

- packet_id: unique identifier for the transmitted packet

- device_id: identifier of user module

- latitude: GPS latitude

- longitude: GPS longitude

- message_type: normal/high/emergency/cancel

- signal_type: manual/auto

- event_time: time generated by device

- source: hardware or simulation

### 6.2 Outgoing ACK JSON (Backend -> Base Station)

{
  "device_id": "DEV_001",
  "ack_type": "delivery_ack",
  "for_packet_id": "a1b2c3d4",
  "timestamp": "2026-03-18T10:33:00Z"
}

### ACK Rules

- ACK only for valid manual alerts

- No ACK for auto signals

- No ACK for cancel messages

- No retry logic

- No delivery confirmation tracking in backend

## 7. MQTT Topic Design

Recommended simple topic structure:

fecs/incoming
fecs/ack

### Topic Usage

- fecs/incoming: base station publishes incoming alert messages to backend

- fecs/ack: backend publishes ACK commands to base station

## 8. Duplicate Detection Rule

A message shall be treated as duplicate if all of the following are true: - same device_id - same message_type - same signal_type - event time difference is less than or equal to 10 seconds - GPS distance difference is less than or equal to 10 meters

### Duplicate Handling Policy

- store only the first valid message

- discard all duplicate copies

- do not store duplicate entries

## 9. Backend Processing Flow

### 9.1 Incoming Processing

- Receive JSON message from MQTT topic fecs/incoming

- Validate schema and field values

- Check for duplicate using time and distance rule

- If duplicate, discard message

- If valid, store alert in database

- If valid manual alert, publish ACK command to fecs/ack

- Push alert update to frontend dashboard

### 9.2 Validation Rules

- device_id must not be empty

- latitude must be between -90 and 90

- longitude must be between -180 and 180

- message_type must be one of supported values

- signal_type must be manual or auto

- event_time must be a valid ISO timestamp

## 10. Database Design

### 10.1 Alerts Table

| Field | Type | Purpose |
| --- | --- | --- |
| id | integer / UUID | primary key |
| packet_id | string | packet identifier |
| device_id | string | sender device |
| latitude | float | user latitude |
| longitude | float | user longitude |
| message_type | string | normal/high/emergency/cancel |
| signal_type | string | manual/auto |
| event_time | datetime | device event time |
| received_at | datetime | backend receive time |
| status | string | pending/acknowledged/resolved |
| source | string | hardware/simulation |

### 10.2 Notes

- received_at is generated by backend when data is accepted

- duplicates are not stored

## 11. Frontend Dashboard Design

### 11.1 Main Dashboard Features

- live alert list

- live Leaflet map

- marker display using GPS coordinates

- visual distinction based on message priority

- current status display for each alert

### 11.2 Suggested Views

#### A. Live Dashboard

- active alerts

- map markers

- latest incoming alerts

#### B. Alert Detail Panel

- device ID

- message type

- signal type

- event time

- location

- status

#### C. Alert History

- list of stored alerts

- filter by type and status

### 11.3 Marker Behavior

Suggested marker coloring: - normal -> green - high -> orange/yellow - emergency -> red - resolved -> grey

## 12. Real-Time Behavior

The dashboard must update in real time when a new valid alert is stored.

Recommended implementation options: - WebSocket from FastAPI to React frontend - or periodic polling as fallback

Preferred option: WebSocket

## 13. Unconscious Detection Logic

### Purpose

Detect whether a user may be unconscious when no manual alert is sent.

### Existing Concept

- user module sends auto location signal every 2 hours

- backend/dashboard monitors repeated unchanged locations

- in daytime, stagnation can escalate to high-priority alert

- in nighttime, base station sends buzzer-trigger signal to user module

- if user responds within configured response period, situation is treated as safe

- if no response is received, case escalates to emergency

### Current Note

This feature is part of FECS concept and can be implemented after core alert flow is completed.

## 14. System Responsibilities by Module

### User Module

- detect button input

- send alert packet

- receive ACK through LED/buzzer

### Relay Module

- forward packets toward base station or user module

### Base Station Module

- receive LoRa packets

- convert/forward structured message to backend

- receive ACK command from backend

- transmit ACK through relay path to user module

### Backend

- validate JSON

- remove duplicates

- store valid alerts

- trigger ACK for valid manual alerts

- send updates to dashboard

### Frontend

- display live alerts

- show map and status

- support operator monitoring

## 15. Recommended Development Plan

### Phase 1: Backend Core

- create FastAPI project

- create MQTT subscriber for fecs/incoming

- create validation schema

- create duplicate detection logic

- create DB insert flow

- create ACK publish flow to fecs/ack

### Phase 2: Database

- create PostgreSQL alerts table

- connect backend to database

- test insert and query functions

### Phase 3: Frontend

- create React dashboard

- integrate Leaflet map

- fetch alerts from backend

- show real-time updates

### Phase 4: Integration Testing

- connect simulated messages from base station

- validate duplicate filtering

- validate ACK publishing

- validate live dashboard updates

## 16. Sequence Diagram

### 16.1 Alert Processing Sequence

sequenceDiagram
    participant U as User Module
    participant R as Relay Module(s)
    participant B as Base Station Module
    participant M as MQTT Broker
    participant A as FastAPI Backend
    participant D as PostgreSQL Database
    participant F as React Dashboard

    U->>R: Send LoRa alert packet
    R->>B: Forward packet
    B->>M: Publish incoming JSON to fecs/incoming
    M->>A: Deliver message
    A->>A: Validate payload
    A->>A: Check duplicate rule
    alt Valid non-duplicate message
        A->>D: Store alert record
        D-->>A: Insert success
        A->>F: Push real-time alert update
        alt Manual alert (normal/high/emergency)
            A->>M: Publish ACK command to fecs/ack
            M->>B: Deliver ACK command
            B->>R: Transmit ACK into LoRa network
            R->>U: Forward ACK
            U->>U: Blink LED
        else Auto signal or cancel
            A->>A: No ACK triggered
        end
    else Duplicate message
        A->>A: Discard message
    end

### 16.2 Unconscious Detection Sequence

sequenceDiagram
    participant U as User Module
    participant R as Relay Module(s)
    participant B as Base Station Module
    participant M as MQTT Broker
    participant A as FastAPI Backend
    participant D as PostgreSQL Database
    participant F as React Dashboard

    U->>R: Send auto location signal (2-hour interval)
    R->>B: Forward packet
    B->>M: Publish incoming JSON to fecs/incoming
    M->>A: Deliver message
    A->>A: Validate and store auto signal
    A->>D: Save alert/activity record
    A->>A: Compare with recent location history
    alt Same-location pattern suggests stagnation
        A->>F: Raise stagnation warning
        alt Night-time check
            A->>M: Publish buzzer-trigger ACK/command
            M->>B: Deliver command
            B->>R: Transmit through LoRa network
            R->>U: Forward buzzer trigger
            U->>U: Activate buzzer
            alt User responds in allowed time
                U->>R: Send normal alert response
                R->>B: Forward response
                B->>M: Publish to fecs/incoming
                M->>A: Deliver response
                A->>D: Store response/update status
                A->>F: Mark user safe
            else No response
                A->>D: Escalate case as emergency
                A->>F: Show emergency alert
            end
        else Day-time stagnation
            A->>D: Escalate case as high-priority alert
            A->>F: Show high-priority alert
        end
    else Location pattern normal
        A->>A: No escalation
    end

## 17. Architecture Diagram

### 17.1 Application Architecture

graph TD
    U[User Module
ESP32 + GPS + LoRa + Button + LED/Buzzer]
    R[Relay Module(s)
LoRa Repeater / Forwarding Node]
    B[Base Station Module
LoRa Gateway connected to Laptop/Server]
    M[MQTT Broker
Topics: fecs/incoming, fecs/ack]
    A[FastAPI Backend
Validation + Duplicate Filter + ACK Trigger + Business Logic]
    D[(PostgreSQL Database
Alerts Table)]
    W[WebSocket / API Layer]
    F[React Dashboard
Leaflet Live Map + Alert Views]

    U --> R
    R --> B
    B --> M
    M --> A
    A --> D
    A --> W
    W --> F
    A --> M
    M --> B
    B --> R
    R --> U

### 17.2 Backend Internal Architecture

graph LR
    M[MQTT Subscriber] --> V[Payload Validation]
    V --> X[Duplicate Detection]
    X -->|Valid| S[Alert Service]
    X -->|Duplicate| G[Discard]
    S --> D[(PostgreSQL)]
    S --> K[ACK Publisher]
    S --> N[Real-Time Notifier]
    K --> MQ[MQTT Topic: fecs/ack]
    N --> WS[WebSocket/API]
    WS --> UI[React Dashboard]

## 18. Final Implementation Statement

This software design converts FECS from a concept into an implementable application architecture. The application is centered around a FastAPI backend, MQTT-based message ingestion, PostgreSQL alert storage, and a real-time React dashboard with map visualization. The backend stores only valid non-duplicate alerts and triggers ACK only for valid manual alerts through the base station module.

The sequence diagrams define how alerts move through the system and how acknowledgments return to the user module. The architecture diagrams define the software building blocks needed for your team to begin implementation.